# ARK-5 Navigation Tasks - Implementation Report

TASK-01 to TASK-12 - Python / YAML only, existing hardware

This report documents how each task in TEAM_NAVIGATION_TASKS.pdf was implemented on top of the ARK_5_Asad code base. Nothing under esp32/ or wiring/ was changed, no LiDAR/encoders were added, and every change is on the laptop side (Python files and config/project.yaml).

# 1. Summary

| Task | Title | Domain | Status |
| --- | --- | --- | --- |
| TASK-01 | Sonar (Ultrasonic) Median Filter | D1 | Done + tested |
| TASK-02 | Yaw Jump Guard | D1 | Done + tested |
| TASK-03 | Honest Dead Reckoning | D1 | Done + tested |
| TASK-04 | Occupancy Grid from Ultrasonics | D2 | Done + tested |
| TASK-05 | Visited Cells + Frontiers | D2 | Done + tested |
| TASK-06 | A* Planner | D3 | Done + tested |
| TASK-07 | FSM: Path Follow + Junctions | D4 | Done + tested |
| TASK-08 | Stuck / Oscillation Watchdog | D4 | Done + tested |
| TASK-09 | Unique Human Tracker | D5 | Done + tested |
| TASK-10 | WiFi RSSI as Hint, Not Brake | D5 | Done + tested |
| TASK-11 | Swarm Wiring Fix | D5 | Done + tested |
| TASK-12 | Demo HUD / Viva Output | D5 | Done + tested |

Test result: 45 of 45 automated checks pass (python tests/test_navigation_tasks.py).

Hardware: esp32/ and wiring/ verified identical to the original zip.

Extra bug fixed: terminal_hud.py's render_swarm() read r.last_v and r.last_omega, which were never set anywhere, so the swarm dashboard would crash. They are now declared and assigned.

# 2. How the pieces fit together

Data flow per control tick (single robot and swarm use the same idea):

- Raw UDP telemetry -> sonar median filter + yaw jump guard (TASK-01/02).

- Filtered sonar rays -> occupancy grid, robot cell marked visited (TASK-04/05).

- Nearest frontier -> A* path -> next waypoint heading (TASK-05/06).

- FSM cruises with corridor centering and the waypoint heading, handles junctions and creep safely (TASK-07).

- Watchdog can override (v, omega) if the robot is stuck (TASK-08).

- Honest pose update from the final (v, omega) and sonar trend (TASK-03).

- Detections go through the unique human tracker; WiFi is only a hint unless confirmed (TASK-09/10).

- HUD prints state, plan, frontier and unique humans (TASK-12); swarm merges both grids (TASK-11).

# 3. Task details

## TASK-01 - Sonar (Ultrasonic) Median Filter  (D1)

Files: NEW utils/sensor_filter.py; hooked into main_controller.py and swarm_controller.py

Problem: Raw ultrasonic values of -1 (no echo), 2 cm glitches and 400 cm spikes went straight into the FSM, causing false stops and false 'open path' decisions.

What was done: SonarMedianFilter keeps the last 5 samples per sensor, returns the median, ignores values <= 0 and rejects jumps larger than 80 cm from the last accepted value. Front, left and right each have their own filter.

Verified by: Feeding [45, 46, -1, 44, 2, 47, 400, 45] never produces -1, 2 or 400 in the output.

## TASK-02 - Yaw Jump Guard  (D1)

Files: utils/sensor_filter.py

Problem: The log showed yaw jumping from 0 deg to -173 deg within one second, which breaks pose and turns.

What was done: YawJumpGuard unwraps the angle and caps the change to about 90 deg per second (scaled by elapsed time), so a bad IMU sample can never teleport the heading.

Verified by: A raw jump of -173 deg in 1 s is limited to a 90 deg step.

## TASK-03 - Honest Dead Reckoning  (D1)

Files: utils/tentative_map.py (update_pose)

Problem: Map position was integrated from commanded speed x time even when the robot was paused, in failsafe, or stuck, so the pin walked across the map during ALERT_PAUSE.

What was done: (x, y) now only advances if failsafe is off, |v| > 0, AND the front sonar trend agrees with the motion (distance shrinks going forward / grows going backward). Otherwise the pose is frozen for that tick.

Verified by: Pose stays frozen for v = 0, failsafe, and when distance grows while 'moving forward'; it advances when the sonar confirms motion.

## TASK-04 - Occupancy Grid from Ultrasonics  (D2)

Files: NEW utils/occupancy_grid.py

Problem: The ASCII dot map is for reading, not planning; wall glyphs were painted inaccurately.

What was done: 5 cm cell grid. Along each valid sonar ray cells are marked FREE and the hit cell OCCUPIED, only for readings between 15 and 300 cm. Front, left (+90 deg) and right (-90 deg) are used. UNKNOWN is kept separate.

Verified by: Readings of 5 cm and 350 cm mark nothing; a 100 cm reading marks the wall at 1.0 m and free cells before it.

## TASK-05 - Visited Cells + Frontiers  (D2)

Files: utils/occupancy_grid.py

Problem: The robot could loop the same corridor forever because nothing remembered visited space.

What was done: The robot's own cell is marked VISITED. Unknown cells adjacent to free/visited cells are FRONTIERS, and the nearest frontier becomes the next exploration goal.

Verified by: Frontiers appear next to explored free space and nearest_frontier() returns a goal.

## TASK-06 - A* Planner  (D3)

Files: NEW utils/astar.py

Problem: There was no real path planning, only rotate-and-guess.

What was done: A* on the occupancy grid (8-connected, obstacles inflated by 1 cell) returns waypoints. If no path exists it returns None so the FSM falls back to its normal scan behaviour. An iteration cap prevents hangs.

Verified by: Finds a path through a gap in a wall; returns None cleanly when the wall is fully sealed.

## TASK-07 - FSM: Path Follow + Junctions  (D4)

Files: utils/autonomous_fsm.py; config/project.yaml (control:)

Problem: Left-always-wins junctions, scan early-exit, and creep getting stuck at about 50 cm while spinning.

What was done: CRUISE does corridor centering from the left/right sonar difference and blends in the A* waypoint heading. WIFI_SCAN looks at both sides before choosing. SCAN_ROTATE always completes the full +/-90 deg sweep. CREEP_TO_SCAN aborts after 3 s. Alignment tolerance tightened from 8 to 4 deg.

Verified by: Six checks: alignment 4 deg, centering direction, heading blend, better side chosen (right 250 cm over left 130 cm), creep timeout, no early exit.

## TASK-08 - Stuck / Oscillation Watchdog  (D4)

Files: NEW utils/watchdog.py

Problem: Left-right wobble and state ping-pong could freeze the robot for minutes.

What was done: If the pose barely changes for about 4 s while a wall is present, a recovery turn opposite to the exploration bias is forced. If the same FSM state lasts more than 8 s, it is reset to CRUISE (ALERT_PAUSE is exempt).

Verified by: Fires when stuck near a wall, resets a stale state, and does not trigger while the robot is genuinely moving.

## TASK-09 - Unique Human Tracker  (D5)

Files: NEW utils/human_tracker.py; main_controller.py and swarm_controller.py

Problem: One person was logged 18 times at almost the same coordinate because the cooldown was time-only.

What was done: A new pin is created only if no existing track lies within about 80 cm. Priority is standing > fallen > person > wifi, so WiFi never downgrades a confirmed detection but a CV detection can upgrade a WiFi guess. Each track stores id, class, confidence, x, y and timestamp.

Verified by: 18 detections of one person collapse to 1 pin; a person 3 m away gets a new pin; priority rules hold.

## TASK-10 - WiFi RSSI as Hint, Not Brake  (D5)

Files: utils/wifi_sensing.py; main_controller.py; swarm_controller.py; utils/tentative_map.py; config/project.yaml

Problem: An RSSI dip could halt the robot and falsely mark humans.

What was done: WiFi alone only draws a '?' hint on the map and does not stop the motors or add to the human count. The motors stop only if YOLO also confirms, or there are 3 consecutive drops AND a wall ahead. The consecutive-drop counter resets on any normal sample.

Verified by: Streak counting and reset verified; hints do not change human_count while confirmed detections do.

## TASK-11 - Swarm Wiring Fix  (D5)

Files: swarm_controller.py

Problem: swarm_controller.py always passed dist_right = -1 and wifi_alert = None, so Robot 2 believed its right side was blocked and never used WiFi sensing.

What was done: The real right sonar is read from telemetry and filtered; WiFi sensing is enabled per robot with the same gating as single-robot mode; a yielding robot waits at most 1.5 s then replans; both robots' occupancy grids are merged into maps/swarm_combined_occupancy.txt (OCCUPIED wins).

Verified by: Hard-coded values are gone from the source; merge keeps walls seen by either robot.

## TASK-12 - Demo HUD / Viva Output  (D5)

Files: utils/terminal_hud.py; main_controller.py

Problem: Logs and the ASCII map did not look like a finished project.

What was done: The HUD now shows filtered ultrasonics, yaw, FSM state, the active A* path length, the unique human count and the next frontier goal. The map legend now includes '?' for WiFi hints.

Verified by: New HUD parameters exist and the dashboard is wired from the main loop.

# 4. New configuration keys (config/project.yaml)

| Key | Default | Purpose |
| --- | --- | --- |
| control.align_tolerance_deg | 4.0 | Turn alignment tolerance (was 8) |
| control.creep_timeout_s | 3.0 | Abort creep if standoff not reached |
| control.center_gain / center_omega_max | 0.0035 / 0.15 | Corridor centering strength and cap |
| control.path_gain / path_omega_max | 0.02 / 0.25 | A* heading-follow strength and cap |
| navigation.sonar_filter_window | 5 | Median window size |
| navigation.sonar_max_jump_cm | 80 | Spike rejection threshold |
| navigation.yaw_max_deg_per_s | 90 | Yaw rate cap |
| navigation.occupancy_cell_m | 0.05 | Planner grid cell size |
| navigation.replan_interval_s | 1.5 | A* replanning period |
| navigation.watchdog_pose_timeout_s / state_timeout_s | 4.0 / 8.0 | Watchdog thresholds |
| navigation.human_merge_radius_m | 0.8 | Same-person merge radius |

# 5. How to run and check

- Automated checks: cd human_detector_robot, then python tests/test_navigation_tasks.py

- Single robot without motors: python main_controller.py --no-motors

- After a run, open maps/robot1_movement_log.txt: there should be no yaw teleport, no repeated human pins and no endless CREEP.

- Swarm run writes maps/swarm_combined_map.txt and maps/swarm_combined_occupancy.txt.

# 6. Honest limitations

All logic was verified with unit tests and a simulated control loop, not on the physical robots (no hardware was available). Values such as centering gain, path gain and the sonar mounting angles (left = +90 deg, right = -90 deg) are sensible defaults and may need small tuning on the real robots; they are all in project.yaml.