# ARK-5 Human Detector Robot

Code Improvements — Detailed Fix Documentation

This document details the 7 improvements made to human_detector_robot (the laptop-side control software). For each fix it explains: which file was affected, what the problem was before, what would have gone wrong in the real world if it had been left unfixed, and exactly what was changed.

# Summary Table

| # | Fix | File(s) |
| --- | --- | --- |
| 1 | Camera stream lag — threaded frame grabber | utils/vision.py |
| 2 | Skip AI inference on every frame (frame-skip) | config/project.yaml, main_controller.py |
| 3 | Detection thresholds centralized in config | config/project.yaml, main_controller.py |
| 4 | Telemetry staleness detection + failsafe stop | utils/udp_client.py, main_controller.py, config/project.yaml |
| 5 | Auto-reconnect on camera disconnect | utils/vision.py |
| 6 | Replaced print() with proper logging | main_controller.py, utils/vision.py, utils/udp_client.py |
| 7 | Config validation for missing/invalid settings | utils/config_validation.py (new file), main_controller.py |

# Detailed Fixes

## Fix #1 — Camera Stream Lag — Threaded Frame Grabber

File: utils/vision.py

What the code looked like before (the problem):

open_stream() returned a plain cv2.VideoCapture object, and the main loop called cap.read() directly — a blocking call.

What this actually caused:

The ESP32-CAM sends an MJPEG video stream over WiFi. If the network was even slightly slow or the CPU was busy, old frames would pile up in the capture buffer. By the time the program processed those buffered frames, the robot had already physically moved past that point — meaning the robot was making decisions based on a 'stale' picture instead of its actual current position.

What would have gone wrong if this was left unfixed:

The robot would react late — for example, a wall or person would only be detected once it was already very close, by which point it could be too late to stop in time. This is a serious problem for a real-time control loop.

What was changed:

A new ThreadedCameraStream class was added that continuously grabs new frames from the camera in a background thread and keeps only the MOST RECENT frame in memory. Whenever the main loop calls read(), it immediately gets the latest available frame — stale frames are automatically discarded. This same class also handles disconnect/reconnect (see Fix #5).

How it was verified:

Tested by running against a video/webcam source and confirming the frame delay dropped noticeably; also verified that throttling the network no longer caused the loop to get stuck processing an old frame.

## Fix #2 — Skip AI Inference on Every Frame (Frame-Skip)

File: config/project.yaml, main_controller.py

What the code looked like before (the problem):

detect_humans() (the YOLOv8 model) was being called on EVERY single frame, regardless of whether the laptop had a GPU or was running on CPU only.

What this actually caused:

Running a deep-learning model like YOLO on every frame on a CPU is slow. This dragged down the overall loop's FPS (frames per second), making the camera preview feel laggy and delaying the robot's decisions as well.

What would have gone wrong if this was left unfixed:

On a lower-spec laptop, the entire system would slow down — the camera preview would stutter, and even fast-reacting sensors like ultrasonic/WiFi would end up being processed late because they were stuck behind the slow AI loop.

What was changed:

A new setting, yolo.infer_every_n_frames (default 2), was added to config/project.yaml. main_controller.py now only runs a fresh YOLO detection every Nth frame; in between frames it reuses the most recent result. A gap of 2-3 frames is imperceptible to the human eye (well under 0.1 seconds), but it noticeably reduces CPU load.

How it was verified:

Manually traced the frame_idx counter and modulo check, and confirmed that setting N=1 reproduces the original every-frame behavior exactly — so it's backward compatible and won't break anyone who wants the old behavior.

## Fix #3 — Detection Thresholds Centralized in Config

File: config/project.yaml, main_controller.py

What the code looked like before (the problem):

Inside main_controller.py, the _cv_alert_message() function had confidence numbers (0.4, 0.5, 0.55) hardcoded directly in the code, while config/project.yaml already had a separate yolo.conf: 0.4 setting.

What this actually caused:

Keeping numbers in two places meant that if someone changed the value in the config file (e.g. to tune accuracy), the old hardcoded number in the code would stay exactly the same — creating a silent mismatch that would be very hard to notice.

What would have gone wrong if this was left unfixed:

If a team member later tuned thresholds by editing only the yaml file (the normal expectation), their change simply wouldn't take effect, because the real number was hidden elsewhere in the code — this would waste hours of confused debugging.

What was changed:

Three separate settings were added to config/project.yaml: fallen_conf, standing_conf, and person_conf. _cv_alert_message() now receives these values as a parameter straight from the config dictionary — there are no hardcoded numbers left in the code.

How it was verified:

Verified in main() that the cv_thresholds dictionary correctly pulls its values from config; also confirmed with py_compile that the syntax is valid.

## Fix #4 — Telemetry Staleness Detection + Failsafe Stop

File: utils/udp_client.py, main_controller.py, config/project.yaml

What the code looked like before (the problem):

The poll_telemetry() function, whenever called, would return the last-known telemetry if no new UDP packet had arrived — without ever indicating how old that data actually was.

What this actually caused:

If the robot's WiFi link dropped for any reason (out of range, interference, ESP32 crash), the laptop-side code had no way of noticing. It would keep making decisions based on stale distance/yaw values — for example, treating 'the path was clear 5 seconds ago' as still true, and continuing to send the robot forward, even though no one actually knew what was in front of it anymore.

What would have gone wrong if this was left unfixed:

This was a genuine safety issue: even after the link dropped, the robot could keep moving 'blind', potentially hitting a wall or driving into a hazardous area — because the system had no way of knowing the sensor connection had been lost.

What was changed:

A last_telemetry_time field and a seconds_since_telemetry() method were added to the UdpMotorClient class, reporting how long ago fresh data last arrived. main_controller.py reads a new telemetry_timeout_s setting from config (default 1.5 seconds) — if no new data arrives within that window, failsafe_active becomes True and the motors are immediately commanded to stop (v=0, omega=0), regardless of what the FSM would otherwise decide. Normal operation resumes automatically once the link comes back.

How it was verified:

Simulated with a small Python test script: sent a UDP packet and confirmed seconds_since_telemetry() immediately reads ~0; then waited 1.6 seconds and confirmed the value rises above the 1.5s threshold — meaning the failsafe will trigger at the right time. This output was verified directly in the terminal.

## Fix #5 — Auto-Reconnect on Camera Disconnect

File: utils/vision.py

What the code looked like before (the problem):

open_stream() only connected to the camera/stream once, at program startup. If the WiFi camera dropped mid-run (ESP32-CAM restart, WiFi glitch), there was no reconnect logic at all.

What this actually caused:

Once the stream failed, cap.read() would just keep returning False forever — no error, no recovery — leaving the robot effectively blind, or causing the whole program to crash.

What would have gone wrong if this was left unfixed:

During field testing (where WiFi hiccups are common), a single brief disconnect could ruin an entire demo/run, requiring the program to be manually restarted from scratch.

What was changed:

A _reconnect() method was added inside the same ThreadedCameraStream class from Fix #1 — if a frame read fails, the class releases the old capture object, waits briefly (2 seconds), and automatically tries to reconnect. All of this happens in the background thread, so the main program never freezes.

How it was verified:

Confirmed via code review that the _loop() method calls _reconnect() on every read failure without crashing the thread; also verified with py_compile that the syntax is valid.

## Fix #6 — Replaced print() With Proper Logging

File: main_controller.py, utils/vision.py, utils/udp_client.py

What the code looked like before (the problem):

The entire project only used Python's print() — no timestamps, no severity levels (INFO/WARNING/ERROR), and output was limited to the console only (nothing was ever saved anywhere).

What this actually caused:

If the robot was running in the field (away from a lab/demo setup) and something went wrong, all of that information was lost forever once the console closed. There was no way to figure out exactly what had happened and when.

What would have gone wrong if this was left unfixed:

Debugging became very difficult — a question like 'the robot suddenly stopped, why?' would be impossible to answer if the console had already been closed by that point.

What was changed:

A proper logger was set up using Python's built-in logging module (via a new _setup_logging() function) that writes to BOTH the console AND a logs/ark5.log file, with a timestamp and severity level (INFO/WARNING) on every line. All print() calls across the project were replaced with the equivalent log.info() / log.warning() calls.

How it was verified:

Verified all affected files with py_compile; manually traced the code to confirm the logs/ folder is created automatically if it doesn't already exist.

## Fix #7 — Config Validation for Missing/Invalid Settings

File: utils/config_validation.py (new file), main_controller.py

What the code looked like before (the problem):

If project.yaml was missing a required setting, or had the wrong type for a value (e.g. text instead of a number), the program would run partway, then crash with a confusing KeyError or TypeError.

What this actually caused:

If a new team member — or even the same person, later on — made a mistake while editing the config file, the error message gave no indication of what was ACTUALLY wrong; it was just a generic Python traceback.

What would have gone wrong if this was left unfixed:

Debugging time would be wasted, especially close to a deadline or right before a demo when a last-minute config change was being made.

What was changed:

A new file, utils/config_validation.py, was created with a validate_config() function. It checks all required settings at startup (camera.stream_url, robot.udp_host, robot.udp_port, robot.telemetry_port, yolo.weights, yolo.conf). If anything is missing or the wrong type, it raises one clear error listing exactly which settings need to be fixed, along with an example of a correct value.

How it was verified:

Ran two test cases: (1) against the real project.yaml — validation PASSED. (2) against a deliberately incomplete config — the 5 missing keys were correctly and clearly reported. Both results were confirmed directly in the terminal output.

# Overall Note

After all seven fixes, the core robot logic — FSM states, wiring, ESP32-side firmware — remains exactly the same as before. Only the laptop-side software has been made faster, more reliable, and easier to debug. All files were syntax-checked with py_compile, and the config/UDP-staleness logic was additionally verified with small automated tests.